CONCEPT DESCRIPTION: PAF Receiver Assembly

Document number	WP2-XXXXXXXXXXXXXXXX
Revision	     
Author	
Date	     
Status	Draft

| Name | Designation | Affiliation | Date | Signature |
| --- | --- | --- | --- | --- |
| Additional Authors |  |  |  |  |
| Submitted by: |  |  |  |  |
| Accepted by: |  |  |  |  |
| Approved by: |  |  |  |  |

DOCUMENT HISTORY

| Revision | Date Of Issue | Engineering Change Number | Comments |
| --- | --- | --- | --- |
| A | - | - | First draft release for internal review |
| B | - | - |  |
| C | - | - |  |

DOCUMENT SOFTWARE

|  | Package | Version | Filename |
| --- | --- | --- | --- |
| Wordprocessor | MsWord | Word 2007 |  |
| Block diagrams |  |  |  |
| Other |  |  |  |

ORGANISATION DETAILS

| Name | SKA Program Development Office |
| --- | --- |
| Physical/Postal Address | Jodrell Bank Centre for Astrophysics Alan Turing Building The University of Manchester Oxford Road Manchester, UK M13 9PL |
| Fax. | +44 (0)161 275 4049 |
| Website | www.skatelescope.org |

TABLE OF CONTENTS

1	Introduction	7

1.1	Purpose of the document	7

2	References	7

3	Context	9

3.1 	SKA Hierarchy	9

3.2	Role of PAF Receiver Assembly in the Dish Array	10

4	Physical Description	10

4.1	Direct sampling at RF but with switched narrowband anti-aliasing	11

4.2	RF over fibre	12

4.3	Analog-to-digital conversion	13

4.4	Digital Filterbank/Beamformer	14

4.5	Digital transmission within the Filterbank/Beamformer	14

4.6	Digital Optical Transmitter	14

5	Requirements	15

5.1	Functional Requirements	15

5.1.1	Imaging dynamic range	15

5.1.2	Feed flexibility (dishes)	15

5.1.3	Rapid installation	16

5.1.4	Minimising maintenance cost	16

5.1.5	Reliability	16

5.1.6	Electromagnetic compatibility	16

5.2	Non-Functional Requirements	17

5.2.1	Mass manufacture	17

5.2.2	Operating cost	17

5.2.3	Maximising  A/T per unit system cost	17

5.2.4	Maximising survey speed per unit cost (PAFs)	17

5.2.5	System Environmental Requirements	18

5.2.6	System Operational and Quality Requirements	18

6	Technical Progress to Date	18

7	Cost Estimates	18

8	Plans for Further Development	19

LIST OF FIGURES

Figure 1 Dish Array Hierarchy	8

Figure 2 Direct sampling at RF but with switched narrowband anti-aliasing filters	9

Figure 3 Switched narrowband anti-aliasing filters for direct sampling at RF	10

Figure 4 RF over Single-mode Fibre	12

Figure 5 Topology of a cross connected beamformer	13

LIST OF TABLES

Error! No table of figures entries found.

LIST OF ABBREVIATIONS

Ant.	Antenna

CoDR	Conceptual Design Review

DRM	Design Reference Mission

EoR	Epoch of Reionisation

EX	Example

FLOPS	Floating Point Operations per second

FoV	Field of View

LNA	Low Noise Amplifier

Ny	Nyquist

OH	Over Head

OTPF	Observing Time Performance Factor

Ov	Over sampling

PAF	Phased Array Feed

PrepSKA	Preparatory Phase for the SKA

RFI	Radio Frequency Interference

rms	root mean square

SEFD	System Equivalent Flux Density

SKA	Square Kilometre Array

SKADS	SKA Design Studies

SPDO	SKA Program Development Office

SSFoM	Survey Speed Figure of Merit

TBD	To be decided

Wrt	with respect to

# Introduction

This document describes the concept for the PAF receiver sub-system , which has been developed by the PAFSKA consortium and is applicable to the SKA Dish Array.

The PAF receiver system encompasses (ADD DIAGRAM – pedestal electronics, coarse filterbanks, inter component connects etc).

Many elements of the PAF receiver sub-system are under development by PAFSKA partners for use in new or upgraded radio telescopes, e.g. Apertif (ASTRON), CSIRO (ASKAP), …

These new PAF-based systems provide direct input to the SKA PAF concepts described here. In addition there is direct SKA-related R&D ongoing within the contributing partners as also described in the details which follow.

## Purpose of the document

The purpose of this document is to describe the sub system, including the following information.

- Its context within the Dish Array Element

- Discussion of the SKA requirements that the sub system will address

- Physical description of the sub system

- Target specifications

- Description of interfaces

- Details of technical progress to date

- Cost estimates for production in SKA quantities

- Details of further plans up to production readiness

# References

- SKA Science Case

- The Square Kilometre Array Design Reference Mission: SKA-mid and SKA-Lo v 0.4

- Science Operations Plan

- System Interfaces

- Environmental requirements (natural and induced)

- SKA strategies and philosophies

- Risk Register

- Requirements Traceability

- Logistic Engineering Management Plan (LEMP)

- Risk Management Plan (RMP)

- Document Handling Procedure

- Project Dictionary

- Strategy to proceed to the next phase

- WP3 SKA array configuration report

- WP3 SKA site RFI environment report

- WP3 Troposphere measurement campaign report

- SKA Science‐Technology Trade‐off Process (WP2‐005.010.030‐MP‐004)

- SKA Monitoring and Control Strategy WP2-005.065.000-R-001 Issue Draft E

- “The Square Kilometre Array”, Peter E. Dewdney, Peter J. Hall, Richard T. Schilizzi, and T. Joseph L. W. Lazio, Proceedings of the IEEE Vol. 97,No. 8, August 2009

- System Engineering Management Plan (SEMP) WP2-005.010.030-MP-001Reference 3

- SKA System Requirement Specification (SRS)

- SKA IP Policy Document

# Context

## 3.1 	SKA Hierarchy

The SKA Systems Engineering Management plan has defined multiple layers of hierarchy:

L7: SKA User

L6: System

L5: Element

L4: Sub-System

L3: Assembly

L2: Component

L1: Part

Although not explicitly stated in the SEMP, the hierarchical approach has the advantage of breaking down the complexity of the system. Each layer is only concerned about its own functionality and its interface to the immediately adjacent layers.

Within the hierarchical scheme, the Dish Array is defined at the element level deriving its requirements directly from a subset of System level requirements.  In turn, the sub-system level allows the Dish Array element to be partitioned further into Level 4 functionality, comprising the Dish, PAF and Single Pixel Feed sub systems. Single Pixel Feeds are further divided into Feed Payload and Receiver assemblies at level 3. Introducing these layers of hierarchy ensures that the complexity of the system is broken down such that an individual layers only have to deal with their relevant perspective of the system.

Figure 1 Dish Array Hierarchy

This document describes one family of options for the level 3 system assembly, the PAF receiver.

## 3.2	Role of PAF Receiver Assembly in the Dish Array

This concept is applicable in the case where PAF receivers are implemented as part of the SKA’s dish feed array.

The PAF receiver with RF-over-fibre signal transmission of the signals is a receiver assembly that can be used with a variety of phased array feed sub systems delivering multiple beams, wide FoV, high survey speed.   RF signals from a number of antennas could be sent either to the central site, or to centralised RFI shielded “bunkers”, where the signals are digitised and digitally processed.   The PAF receiver with RF-over-fibre signal transmission specifically addresses the functional and non-functional requirements for a high survey speed for the SKA; many of these requirements are unique and go far beyond what has previously required for ground based radio astronomy.

# Physical Description

Provide a description of the proposed sub system, with diagrams, CAD drawings, photographs etc. Include details of how the sub system will interface with other parts of the Dish Array.

This PAF receiver assembly concept includes the following components;

- A set of two switched anti-aliasing filters to cover the whole 600 – 1500 MHz RF band,

- Transmission of the RF over fibre to the Analog-to-digital conversion block,

- Analog-to-digital conversion using the EV8AQ160 or similar,

- Digital processing in the Filterbank/Beamformer, and

- Digital transmission within the Filterbank/Beamformer using optical fibre.

Figure 2 is a block diagram showing the key components and where they will be housed.

Figure 2 Direct sampling at RF but with switched narrowband anti-aliasing filters

A feature of this option is that it allows digitisation to occur up to, say, 10 km from the antenna.   Thus the digitisation and digital processing of the signals from the antennas in the “core” and “inner” regions (for SKA Phase 1) could be at the central site; for antennas in the “Mid” region, the digitisation and digital processing of signals from one or more “clusters” of antennas could be centralised in RFI shielded “bunkers” and the resulting data trunked to the central site.

In the sections which follow we outline the component descriptions.

## Direct sampling at RF but with switched narrowband anti-aliasing

Figure 3 Switched narrowband anti-aliasing filters for direct sampling at RF

If the specification is for an IF bandwidth of the order of one half or one third of the RF bandwidth, we can use slower digitisers and reduce the amount of digital processing by processing only part of RF band at one time.   We would filter and then sample the whole RF band using a number of switched, narrowband, anti-aliasing filters.

Note that the poly-phase filter that follows the ADC must operate at the clock rate of the ADC but at the output of the filterbank the frequency channels to be processed in the beamformer and correlator can be selected.  In this design anti-aliasing filters with a relaxed specification for the transition band are used.  This allows low cost filters to be used.   After the filterbank frequency channels that are corrupted due to aliasing or excessive band slope can be discarded.  The data at these frequencies is discarded at the output of the filterbank.

For a 600 – 1500 MHz RF band and an IF bandwidth of 400/600 MHz, we could use two switched narrowband anti-aliasing filters to cover the whole RF band and sample rates of 1100 or 1650 MS/s as listed in Table 1a.   Alternatively, to achieve a wider IF bandwidth of 500/600 MHz, we could use the narrowband anti-aliasing filters and sample rates of 2500 or 1650 MS/s as listed in Table 1b.

| RF Band | RF Band (MHz) | RF Bandwidth (MHz) | Sample rate (MS/s) |
| --- | --- | --- | --- |
| Band 1 | 600 – 1000 | 400 | 1100 |
| Band 2 | 900 – 1500 | 600 | 1650 |

Table 1a.	Anti-aliasing filters required to cover the whole 
600 – 1500 MHz RF band using two switched filters

| RF Band | RF Band (MHz) | RF Bandwidth (MHz) | Sample rate (MS/s) |
| --- | --- | --- | --- |
| Band 1 | 600 – 1100 | 500 | 2500 |
| Band 2 | 900 – 1500 | 600 | 1650 |

Table 1b.	Anti-aliasing filters required to cover the whole 
600 – 1500 MHz RF band using two switched filters

With both of these options, the full 600-1500 MHz band is covered, but as the bandwidths of the two bands differ this may lead to under-utilisation of correlator and beamformer resources when band 1 is operational.   For a wideband survey one ‘efficiency’ strategy might be to discard the lowest 100 MHz of band 2 data, and for HI surveys the uppermost 50 MHz .  This reduces the total bandwidth in the beamformer and correlator to 450 MHz.

## RF over fibre

RF over fibre solutions have existed for 30 years. The great challenge for the PAF is production of high stability, high spurious free dynamic range (SFDR) links with low input NF that can be manufactured within the budget.  For SKA we suggest the target needs to be below  AUD 200 per link including connectors.

Due to cost constraints Direct Modulation (DM) Direct Detection Intensity Modulation schemes using high spectral quality, low relative intensity noise (RIN),  Direct Feedback (DFB) Single Mode Lasers (SML) at 1310 nm are seen as the best option for the optical transmitter. High linearity InGaAs/InP PIN diode photodetectors are used as the optical to electrical transducer.

SM Fibre loss at 1310nm zero dispersion wavelength is typically 0.35 dB/km with standard G652 9/125 um fibre.   A link SFDR of 110 dB/Hz2/3 with 0dB optical loss is achievable, this realises an SFDR of 52 dB over a 500 MHz band.   Band transmission should be limited to sub-octave bandwidths, requiring bandpass preselection prior to the RFoF link, minimising generation of link 2nd order intermodulation products.   Links may be extended to 10km (extra 3.5dB optical loss) with only marginal (7dB) reduction in dynamic range. The single mode architecture enables remote placement of the ADC and digital electronics up to 10km away from the antenna receptors greatly mitigating the risk of self RFI and reducing resident antenna hardware.

Bias

SMF 0dB

PreAmp

PostAmp

600-1500MHz

Duoband

Z= 50

Link NF < 20dB

SFDR > 110dBHz2/3

Link G > 0dB

Flatness +/-  1dB

Figure 4 RF over Single-mode Fibre

Single mode transmission of RF requires good Optical Return Loss (ORL), low back reflection connectors. Due to the large number PAF elements mass termination Multiple Parallel Optical (MPO) connectors are preferable. These can be used in 12 wide MPO configurations with Angled Physical Contact (APC) high ORL ~ 60dB properties designed for single-mode ribbon fibre.

By 2017 the cost of the RF over fibre system is expected to be <AUD 200 per link including the post detection amplifiers and filters. This may also include new vertical cavity surface emitting lasers (VCSEL) based devices for 1310nm (and 1550nm) replacing DFB devices.

## Analog-to-digital conversion

Examples of analogue to digital converters that meet the specification for a PAF are the EV8AQ160 and EV10AQ190.   Critical specifications for these are list below.   Converters from other manufacturers are also suitable and as time progresses it is quite likely more options will become available.

For the example EV8AQ160 Analog-to-digital converter the critical specifications are:

- 8-bit;

- 7.2 ENOB;

- 4 samplers interleaved in one chip;

- Max. Sample rate in dual channel mode: 2.5 GS/s;

- Max. input Freq 2 GHz (typ.), 1.5 GHz (min.)

| Max. Sample rate (MHz) | Input bandwidth (MHz) | interleaved |
| --- | --- | --- |
| 1250 | 600 | 1 |
| 2500 | 1200 | 2 |
| 5000 | 2000 | 4 |

Table 2.	EV8AQ160 Analog-to-digital converter

## Digital Filterbank/Beamformer

The Digital Beamformer may include a poly-phase filter bank and processing to reformat the data prior to optical transmission from the antenna pedestal to the central site.

For the Phase 1 SKA beamformer, multiple processing boards are needed with cross connections to do the FX beamforming.

The analog data is digitised and then processed by a coarse filterbank.  Each coarse filterbank system has a direct connection to each of the beamformer boards and for N beamformer boards the data on each connection carries data for 1/N of the frequency channels being processed.

Figure 5 Topology of a cross connected beamformer

## Digital transmission within the Filterbank/Beamformer

Digital data would be transported within the Filterbank/Beamformer shielded enclosure on optical fibre.   Currently the cheapest method of transporting digital data over short spans, ie very short reach (VSR) on fibres is with the use of 12 fibre optical cables illuminated by 12 element optical transmitters such as the industry standard SNAP12 modules [SNAP12 specifications http://www.physik.uzh.ch/~avollhar/snap12msa_051502.pdf].   These are well suited to massively parallel data transfer applications such as BF to COR connects.

SNAP12 modules are expected to fall to under $100 each by 2017 (especially in the volumes required by SKA). Modal dispersion would limit the distance of these inherently multimode 50um core 850nm wavelength devices based on cheap VCSEL optical transmitters.

The use of appropriate fibre such as OM4 Corning Infinicor eSX+ extends the Effective Modal Bandwidth (EMB) from 500 MHz.Km to 4700 MHz.Km.

## Digital Optical Transmitter

Beamformed outputs from the RFI shielded “bunkers” will the trunked to the central site using longer distance variants: 10BASE-ZR at 1550nm that will be capable of 80 km in standard G652 fibre. Again by 2016 we estimate that this will be available for ~AUD 2000 each (in volume).

The Data formatter for the output optical signal will be included in the Digital Beamformer.   The Digital Optical Transmitter includes only the electrical-to-optical transducer and output optical connectors.

It is not clear how this overlaps with Signal Transmission and Networks (STaN) responsibilities.

# Requirements

This section describes how the proposed sub system will address the requirements for the Dish Array, which are derived from the system requirements and ultimately the science requirements. These include both the functional and non-functional requirements. Some of the key requirements that need to be addressed are as follows.

- Imaging dynamic range

- Mass manufacture

- Operating cost

- Feed flexibility (dishes)

- Rapid installation

- Maximising  A/T per unit system cost (i.e including signal transport, signal processing, computing etc.)

- Maximising survey speed per unit cost (PAFs)

- Minimising maintenance cost

- Electromagnetic compatibility

## Functional Requirements

Describe which	requirements are being addressed and how the proposed sub system is expected to meet them. For example, how will the octave band feed payload optimise the Dish Array pulsar observations capability, or how will the proposed PAF option optimise HI surveys?

Input/advice from Lisa?

### Imaging dynamic range

High imaging dynamic range for PAFs is to be demonstrated, but is largely independent of the PAF receiver assembly concept.

Are there any compromises to DR which the receiver might implement (number of bit samplings?)

### Feed flexibility (dishes)

Feed flexibility is not applicable to the PAF receiver assembly concept.

Agree, but perhaps more important to outline which components of the receiver system can be housed where & what flexibility there is (if any) of what to put where.

### Rapid installation

Rapid installation is not applicable to the PAF receiver assembly concept.

No- but it is important that PAF receivers have accessible components which don’t take 4 days to extract one failed part. Modularity? Murandi ideas?

### Minimising maintenance cost

The PAF receiver assembly will be designed to minimise maintenance costs.

How? What are the major traps we know of, is this very costly to mitigate?

What about looking at the closest commercial PAF (defence) who have solved this problem?  (e.g. L3 products)

Do we put more $’s into expensive parts up-front? Build more self-test systems? Diagnose whilst the system is in the pedestal rather than having to remove cards/racks?

Tolerance to environment – heat, cold? Water/dew? (see 5.2.5 System Environmental Requirements)

Graceful degradation?

### Reliability

The entire SKA receiver system will be field repairable to the level of tested sub-assemblies. No manufacturing, assembly or installation processes should be employed that would preclude field replacement of sub-assembly level systems or components.

The antenna reliability is defined by the observatory uptime requirement: no more than 1%?? of the antennas (of the complete SKA array) offline at any one time.  This translates to a MTBF of 20000?? hours for the receiver system. The time to repair any single antenna fault should not exceed one day.

The SKA receiver system should be designed to require little hands-on maintenance. The interval between site visits by a technician to carry out any preventative or routine maintenance activity (including inspections) should not be less than 12 months ??? .

### Electromagnetic compatibility

The PAF receiver assembly will be designed for electromagnetic compatibility.

And…?

What about mitigating self-interference? What are the noisy sources in this system? How can they be tested? Are they reliable or do they vary by T?

The SKA site has been chosen as it is virtually free of any terrestrial radio frequency interference    The SKA receiver assemblies are to be designed to preserve the RFI environment at the SKA site, and this requires that all sources of RFI are to be shielded and made RFI-tight.   As far as possible all components of the receiver are to be housed in RFI-tight cabinet(s).  The cabinets are to meet the requirements of RFI/EMC shielding to provide electromagnetic attenuation better than 80 dB between 10 kHz and 1 GHz, tested as per IEEE-STD-299.   All components of the receiver are to be certified by a qualified testing laboratory to meet the MIL-STD-461F standards ( RE102; In figure RE102-4 the limits for "Navy Mobile and Army" are applicable) over the frequency range 0.3 to 1.8 GHz.

## Non-Functional Requirements

There are many non-functional requirements that will need to be met if the Dish Array is to operate successfully; a few of these are listed above. Describe here how some of the critical requirements are expected to be met. For example, how will it be mass-manufactured to minimise cost, whilst maintaining reliability; how will the maintenance cost be kept to a minimum?

### Mass manufacture

The PAF receiver assembly will be designed to be mass-manufactured.

ASTRON has much experience and many ideas to include here.

What about Murandi’s ideas?

Is there a case to chose one receiver architecture over another at this point? Is it easier to build I rather than 2?

### Operating cost

The PAF receiver assembly will be designed to minimise operating cost.

Power? Which parts/components are power hungry? Temperamental?

The challenging conditions at the SKA site put a premium on minimising operational power consumption.

### Maximising  A/T per unit system cost

The PAF receiver assembly will be designed to minimise cost thus maximising A/T per unit system cost. (i.e including signal transport, signal processing, computing etc.)

Indicative figures – we don’t want to quote ASKAP verbatim but how about some projections?

### Maximising survey speed per unit cost (PAFs)

The PAF receiver assembly will be designed to minimise cost thus maximising survey speed per unit cost.

This needs to be well argued at this point  – need Lisa’s  (and others) analysis here.

### System Environmental Requirements

The receiver system will be designed for reliable operation at the site where the normal operating conditions are :

- Ambient T in the range -10 to +55 degrees C.

- The mean daily solar exposure ranges from 30 MJ/m2 (summer) to 13 MJ/m2 (winter).

Lightning protection is to be built into all components of the receiver system.

In average conditions, there is a significant amount of fine airborne dust (sand): all parts of the receiver system are to be suitably designed to prevent ingress into interior areas or mechanical parts, and minimise build-up on external surfaces.

All exposed parts of the receiver system are to be water-tight and vermin proof: In wildlife may destroy any material known to mankind.

### System Operational and Quality Requirements

The challenging conditions at the SKA site put a premium on a number of design considerations for the SKA receiver system:

- Reliable and safe operation consistent with unattended operation.

- Every aspect of the receiver system is to be designed with the site restrictions in mind:  The SKA needs to be able to maintain and repair the receiver equipment with limited engineering resources and personnel.

# Technical Progress to Date

Give a brief description of development work to date, together with critical results where appropriate. More substantial reports on technical progress can be included as appendices.

Summarise PAF receiver technical progress to date viz. development of RF over fibre and direct RF sampling.

CSIRO to do.

Other partners – particular areas to contribute to be identified.

# Cost Estimates

Provide initial cost estimates for manufacture of the proposed sub system in quantities applicable to the SKA phases 1 and 2.

# Plans for Further Development

Describe the plans to further develop the proposed sub system up to the point where it can be mass manufactured. Include time lines and details of resources that will be available to carry out the work.

Summarise plans for further technical development of key technologies viz. development of RF over fibre and direct RF sampling.

Again – to be identified as soon as possible & start assembling this section.