ATNF ATUC MEMORANDUM

To:		ATUC

From:		Warwick Wilson

Date:		2 June 2004

Subject:		Technology Development	 Report

Highlights

- Delivery and installation of the Arecibo Multibeam receiver on schedule in April.

- Installation of a prototype polyphase digital filterbank at Mopra in May.  The system currently has 1024 channels over 256MHz bandwidth on each of two polarisations.  This is a major step forward in the development of the 2GHz bandwidth filterbanks for the ATCA broadband upgrade.

- The joint ICT/ATNF NASA DSN array studies project has produced new concepts for array beam forming and correlation, which have now been adopted by NASA for their prototype DSN array.

- Good prospects for NASA contributing to production of 7mm ATCA receivers, in return for a small amount of DSN tracking time, starting probably mid to late 2006.

Revised schedules

- Some delays in mm-wave receiver production will mean that 3mm will not be available on the ATCA before late July.  Latest update will be given at ATUC meeting.

- 6GHz multibeam receiver installation at Parkes now scheduled for early in the April 2005 observing term.  Jodrell Bank partners have agreed to this schedule.

- Mopra 8GHz spectrometer and new mm-wave receiver for Mopra, with projected frequency coverage up to 115GHz, now scheduled for installation in time for 2005 mm observing session.

## New Projects

The following new projects were approved by the ATNF Projects Board in December 2003 and March 2004.

- All-sky epoch of re-ionisation experiment.

- Delay tracker for the ATCA wideband analogue correlator.

- Pulsar digital filterbank

- GMRT feed development

- Additional 8MHz filters for the Parkes multibeam correlator.

## EU FP6 Collaboration - PHAROS

ATNF and ICT are collaborating in the EU FP6 PHAROS program, aimed at developing focal plane array systems for large antennas, in the frequency range 4 to 8 GHz.  ATNF’s commitment, of approximately 1 FTE, is in the areas of signal processing, MMIC development and software, and is largely common to work being carried out in other projects.

## Emerging Science Program

ATNF has applied for special funding under the CSIRO emerging science program.  In the technology development area, the proposal includes funding for a study of line feeds for cylindrical reflectors and a design study for a focal plane array receiver, nominally for the Parkes telescope.  ATNF expects funding to be available from July 2004 for both of these activities, initially at the 1FTE level in each.

## Advice sought

ATUC's advice is sought on the following issues/questions:

As mentioned above, ATNF’s emerging science program includes a design study for a focal plane array receiver.  This is initially aimed at a conceptual design of a practical and scientifically interesting FPA system for an ATNF telescope.  The 4 to 8 GHz band chosen by PHAROS is seen to be not particularly interesting in terms of the science it could do.  A 12mm FPA for Parkes has been proposed in the past and may be a good prospect.  Are there better alternatives?