# AT LBA DAS Proposal

Dick Ferris   30-Jul-26

The analog signal from each IF is first filtered to suppress noise in adjacent image bands and then digitised by an 8-bit sampler. The resulting 128Msps/64MHz data is processed into one 32MHz or two 16MHz subbands.  These may be further reduced in bandwidth by factors of two, four, eight or sixteen

In the case of the ATCA, and possibly Mopra, the ADC is redundant as 128Msps/64MHz 4bit data from the integral samplers will be used directly.

The narrower bands are tunable in steps of ?1?Hz (t.b.a.).  Tuning oscillator phase may be synchronised once per second to provide 'continuous phase' operation for frequency switching or doppler tracking schemes.

After filtering the selected output(s) are converted to 2-bit format and presented to an S2 interface for recording .  Any subband may be programmed to appear in its normal or inverted spectrum mode, i.e. USB or LSB, as required.  Outputs are also provided for direct connection to an AT Correlator, including a reformatted but unfiltered data stream which may represent the full 64MHz input band, provided an appropriate antialiase filter is fitted.

The central 32MHz acceptance band is flanked on each side by >40dB stop bands extending for another 32MHz, thus easing the specifications of the antialiase filter.  In addition, moving the input from baseband up to the first alias, ie 64-128MHz, greatly eases the specifications of the filter preceding the final mixer in the IF chain.

There remains considerable potential to activate other patterns of output bands across the range if required in future.

A complete system comprises two filter modules, (nominally one for each polarisation) as shown below, plus power supplies, a Data Set to interface to the control computer, plus software to run on same.

The attached diagrams illustrate the primary configurations and the associated filter structure.  The output options shown reflect the constraint of 128Mbps maximum recording rate of a single S2 rack.

## Output Options

| Filter Response | Mode/Data for 2 IFs | Data for 1 IF |
| --- | --- | --- |
| µ § | 64MHz baseband input #1 Continuum, VSOP 32MHz @ 1bit Continuum, line 16MHz @ 2bit 8MHz @ 2bit 4MHz @ 2bit 2MHz @ 2bit Continuum, VSOP 2 * 16MHz @ 1bit VLBA, MkIII & RadioAstron 2 * 8,4,2,1MHz @ 2bit Line plus optional continuum #2 16MHz @ 1bit + B @ 2-bit 8MHz tunable 4MHz tunable 2MHz tunable 1MHz tunable VLBA, MkIII & RadioAstron #2 2 * 8MHz @ 2bit #3 2 * 4MHz @ 2bit tunable 2 * 2MHz @ 2bit tunable 2 * 1MHz @ 2bit tunable | 32MHz @ 2bit 16MHz @ 2bit 8MHz @ 2bit 4MHz @ 2bit 2MHz @ 2bit 2 * 16MHz @ 2bit 2 * 8,4,2,1MHz @ 2bit 16MHz @ 2bit + B @ 2-bit = = = = = = = = |

#1  128MHz data is available on the correlator port only.

#2  The 16MHz subbands may be interchanged at will.

#3  +/- 0.5MHz tuning is possible with some bandedge degradation.

## Filter Configuration

µ §

_______________________________________