# LBA DAS to S2 Interface

R.H. Ferris  11/10/96

## Clocks

The C1 cable obtains the Input Reference Clock, SCLK_REC, and the Input 1Hz Reference, S1HZ_REC, from  the module in slot 2, i.e. DF 1, even if data is being recorded from IFP 2 (slots 4 and 6) only.  If the N2 module in slot 2 should fail it must be replaced by the good module from slot 6 and all recording done from IFP 1.

The Input Reference Clock, SCLK_REC, is always 32MHz regardless of the data rate.

## Configurations

Either or both IFPs may be in use and each may be programmed independently to produce one or two passbands (of possibly differing bandwidth), and correspondingly one or two data streams at the output.  In turn some of these may be intended for monitoring but not recording, so in the following notes and tables only recorded data streams are relevant.

### Notes

The nominal installation configuration is for the L and R (left & right  circular polarisation) IF signals from the antenna to be connected to the IF Processor 1 and IFP 2 inputs respectively.

Each IFP may be programmed to have a different filter configuration and bandwidth(s).  However for the purpose of setting up the S2 recorder all recorded data streams are considered to have the same bandwidth as the widest recorded data stream.  Likewise if  two data streams are recorded for one IF then the S2 mode will provide either zero or two data streams for the other.  In the latter case if only one data stream is actually wanted the presence of data on the remaining UD channels is immaterial.

IF Processor 1 is wired to S2 user data channels 0..3, and IFP 2 to UDC 4..7.  2-bit data streams have the sign bit on an even numbered UD channel and the magnitude bit on the next channel.  Single bit data uses only even numbered UD channels except for 64MHz BW which uses all channels in the group.  If only one data stream is recorded for an IFP it is allocated the lowest numbered UDC pair in its group.

Non-AT data codes such as VLBA (offset binary) and NASA SERC (ternary) have the MSB on an even numbered UD channel and the LSB on the next channel.

Data at rates less than 4Mbps use 4Mbps modes and will be multiply recorded by the S2; the excess bits are ignored on playback.  Similarly eight channel recording always requires at least two transports.  These factors may sometimes mean that one more transport/tape is used than is theoretically necessary, but the finite set of options in both the S2 and the correlator make it unavoidable.

In order to elucidate the S2 mode requirements the table shows a variety of regular passband configurations but is in no way complete.  For example it is truncated after 1MHz bandwidths as all narrower bands (extending down to 62.5kHz) are perforce treated as 1MHz bands when configuring the S2.  Similarly, multiply over sampled bandwidth options and mixed bandwidth options are omitted as they simply re-use S2 configurations already mentioned in lines above.  eg. [(16+1),(32)] requires 32s4-1 as does [(16+16),(16+16)]

Most data recording will be in 2-bit mode, but 1-bit modes are defined in three circumstances. These are : 1) where 2-bit mode for the filter configuration would exceed the 128Mbps capacity of the S2,  2) to provide compatibility with MkIII systems, and 3) where reducing to 1-bit halves the number of tapes required.  Below 64MHz BW data streams are always produced in 2-bit format and '1-bit' mode is effected by choosing an S2 mode which records only the sign bits (ie only even numbered UD channels).

To cover the full range of configurations and bandwidths, a total of 43 distinct S2 modes is required.  Of these 28 are 'non-standard' and of these latter some 19 are totally new at this date.

ctd. 3pp.

### C1 User Data Connections

| IFP 1 | IFP 2 | S2 Mode | Total | Active User Data Channels on C1 |  |  |  |  |  |  |  |  |  | Mode Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B / B0+B1 | B / B0+B1 | btu-q | Mbps | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  | NonStd | New |
| 64 |  | 32x4-1 | 128 | x | x | x | x |  |  |  |  |  |  |  |  |
|  | 64 | 32?4-1 | 128 |  |  |  |  | x | x | x | x |  |  | << | * |
| 32 |  | 32x4-2 | 128 | x | x | x | x |  |  |  |  |  |  |  |  |
|  | 32 | 32?4-2 | 128 |  |  |  |  | x | x | x | x |  |  | << | * |
| 32 | 32 | 32s4-1 | 128 | x |  | x |  | x |  | x |  |  |  | << |  |
| 32 |  | 32x2-1 | 64 | x |  | x |  |  |  |  |  |  |  |  |  |
|  | 32 | 32?2-1 | 64 |  |  |  |  | x |  | x |  |  |  | << | * |
| 16+16 |  | 32x4-2 | 128 | x | x | x | x |  |  |  |  |  |  |  |  |
|  | 16+16 | 32?4-2 | 128 |  |  |  |  | x | x | x | x |  |  | (<<) | (*) |
| 16 | 16 | 32a4-2 | 128 | x | x |  |  | x | x |  |  |  |  | << |  |
| 16+16 | 16+16 | 32s4-1 | 128 | x |  | x |  | x |  | x |  |  |  | << |  |
| 16 |  | 32x2-2 | 64 | x | x |  |  |  |  |  |  |  |  |  |  |
|  | 16 | 32?2-2 | 64 |  |  |  |  | x | x |  |  |  |  | << | * |
| 16+16 |  | 32s2-1 | 64 | x |  | x |  |  |  |  |  |  |  | << |  |
|  | 16+16 | 32?2-1 | 64 |  |  |  |  | x |  | x |  |  |  | (<<) | (*) |
| 16 | 16 | 32?2-1 | 64 | x |  |  |  | x |  |  |  |  |  | << | * |
| 16 |  | 32x1-1 | 32 | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 16 | 32?1-1 | 32 |  |  |  |  | x |  |  |  |  |  | << | * |
| 8+8 | 8+8 | 16x8-2 | 128 | x | x | x | x | x | x | x | x |  |  |  |  |
| 8+8 |  | 16x4-2 | 64 | x | x | x | x |  |  |  |  |  |  |  |  |
|  | 8+8 | 16?4-2 | 64 |  |  |  |  | x | x | x | x |  |  | << | * |
| 8 | 8 | 16?4-2 | 64 | x | x |  |  | x | x |  |  |  |  | << | * |
| 8+8 | 8+8 | 16s4-1 | 64 | x |  | x |  | x |  | x |  |  |  | << |  |
| 8 |  | 16x2-2 | 32 | x | x |  |  |  |  |  |  |  |  |  |  |
|  | 8 | 16?2-2 | 32 |  |  |  |  | x | x |  |  |  |  | << | * |
| 8+8 |  | 16s2-1 | 32 | x |  | x |  |  |  |  |  |  |  | << |  |
|  | 8+8 | 16?2-1 | 32 |  |  |  |  | x |  | x |  |  |  | << | * |
| 8 | 8 | 16?2-1 | 32 | x |  |  |  | x |  |  |  |  |  | << | * |
| 8 |  | 16x1-1 | 16 | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 | 16?1-1 | 16 |  |  |  |  | x |  |  |  |  |  | << | * |

### ctd. next page

### C1 User Data Connections ctd.

| IFP 1 | IFP 2 | S2 Mode | Total | Active User Data Channels on C1 |  |  |  |  |  |  |  |  |  | Mode Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B / B0+B1 | B / B0+B1 | btu-q | Mbps | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  | NonStd | New |
| 4+4 | 4+4 | 8x8-2 | 64 | x | x | x | x | x | x | x | x |  |  |  |  |
| 4+4 |  | 8x4-2 | 32 | x | x | x | x |  |  |  |  |  |  |  |  |
|  | 4+4 | 8?4-2 | 32 |  |  |  |  | x | x | x | x |  |  | << | * |
| 4 | 4 | 8?4-2 | 32 | x | x |  |  | x | x |  |  |  |  | << | * |
| 4+4 | 4+4 | 8s4-1 | 32 | x |  | x |  | x |  | x |  |  |  | << |  |
| 4 |  | 8x2-2 | 16 | x | x |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 8?2-2 | 16 |  |  |  |  | x | x |  |  |  |  | << | * |
| 4+4 |  | 8s2-1 | 16 | x |  | x |  |  |  |  |  |  |  | << |  |
|  | 4+4 | 8?2-1 | 16 |  |  |  |  | x |  | x |  |  |  | << | * |
| 4 | 4 | 8?2-1 | 16 | x |  |  |  | x |  |  |  |  |  | << | * |
| 4 |  | 16x1-1 | 8/16 | x |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 16?1-1 | 8/16 |  |  |  |  | x |  |  |  |  |  | (<<) | (*) |
| 2+2 | 2+2 | 4x8-2 | 32 | x | x | x | x | x | x | x | x |  |  |  |  |
| 2+2 |  | 4x4-2 | 16 | x | x | x | x |  |  |  |  |  |  |  |  |
|  | 2+2 | 4?4-2 | 16 |  |  |  |  | x | x | x | x |  |  | << | * |
| 2 | 2 | 4?4-2 | 16 | x | x |  |  | x | x |  |  |  |  | << | * |
| 2+2 | 2+2 | 4s4-1 | 16 | x |  | x |  | x |  | x |  |  |  | << |  |
| 2 |  | 8x2-2 | 8/16 | x | x |  |  |  |  |  |  |  |  |  |  |
|  | 2 | 8?2-2 | 8/16 |  |  |  |  | x | x |  |  |  |  | (<<) | (*) |
| 2+2 |  | 8s2-1 | 8/16 | x |  | x |  |  |  |  |  |  |  | (<<) |  |
|  | 2+2 | 8?2-1 | 8/16 |  |  |  |  | x |  | x |  |  |  | (<<) | (*) |
| 2 | 2 | 8?2-1 | 8/16 | x |  |  |  | x |  |  |  |  |  | (<<) | (*) |
| 1+1 | 1+1 | 4x8-2 | 16/32 | x | x | x | x | x | x | x | x |  |  |  |  |
| 1+1 |  | 4x4-2 | 8/16 | x | x | x | x |  |  |  |  |  |  |  |  |
|  | 1+1 | 4?4-2 | 8/16 |  |  |  |  | x | x | x | x |  |  | (<<) | (*) |
| 1 | 1 | 4?4-2 | 8/16 | x | x |  |  | x | x |  |  |  |  | (<<) | (*) |
| 1+1 | 1+1 | 4s4-1 | 8/16 | x |  | x |  | x |  | x |  |  |  | (<<) |  |
| 1 |  | 8x2-2 | 4/16 | x | x |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 8?2-2 | 4/16 |  |  |  |  | x | x |  |  |  |  | (<<) | (*) |

1.	B / B0+B1 <=> Nominal passband bandwidth/s in MHz

2.	(<<), (*) : repetitions of non-standard  modes above

3.	8/16 etc.: data/recorder bps

_________________________________________________________________________________________________

file:  s2modes.doc																		   printed  30/07/26 23:13